You will need

- 10, 12,14,& 22mm socket and ratchet

- Flat head screw driver

- Philips screw driver

- 10,12 & 14 Spanners

- Long nose pliers

I will go through how i done the tappets on the front bank first.  The only difference with doing the rear bank is that you have to remove the intake manifold.  While you are doing this you might as well as change the spark plugs.

Front Bank

Step1: Remove the 6 screws on the coil covers and oil cap

Step 2: Remove the 3 coil packs by undoing the 2 10mm bolts on each pack

Step 3: Remove the 9 bolts on the rocker cover

Step4: Remove the top timing belt cover. There are 4 10mm bolts holding this on, 3 of which are on plane view and 1 which is located in the direction of the power steering pumps, this one can be awkward to get at.

Step 5: Lift off the rocker cover. Resist the temptation to wedge off the cover with a screw driver if you plan on reusing the gasket. A gentle tap with a rubber mallet should do the job. The picture below is what you will see.

Step 6: Removing the tappets. The technique i used here was to find valves that are opened, ie (the lobes on the cams are facing up) see picture below. There will be 2 valves open at any one time.

Then stick a flat head screw driver under the rocker arm and above the tappet levering off the rocker arm to the left or the right but be careful not to scratch the lobes.  Cover the area with your free hand as the rocker arm can go flying. The long nose pliers will come in handy here to pick the rocker arm out of those hard to reach areas.

The tappets can now be removed. Slide in the new tappets slowly but make sure that you cover them in engine oil first so that they don’t do any damage when you restart the engine. Replace the rocker arm by sliding it on top of the valve and lever it back onto the tappet. This technique was hard to show in a picture but you will get the hang of it very easily.

Step 7: Opening the rest of the valves

The trick here is to turn the crankshaft a few degrees in the clockwise direction until you see another set of valves open and repeat step 6 above.  Do not turn the engine from the camshaft as this is what i done and the cam slipped few teeth on the timing belt.  I was doing the timing belt anyway so it was that big of a deal for me.

Repeat steps 6 and 7 for all of the valves and tappets in the front bank.

Step 8:  All tappets are now changed so you can refit the rocker cover and coil pack. Remember it’s just as handy to change the spark plugs at this stage. Refitting is the reverse of removal.

Rear Bank

Changing the tappets in the rear bank is the exact same as the front bank with the only difference being that you have to remove the intake manifold which i will take you through and the angles are a little bit harder to work with.

Step 1: Remove the strut brace if you have one.

Step 2: Remove the air intake hose by undoing the two jubilee clips.

Step 3: Remove the throttle body by undoing the 5 12mm bolts, 4 in the front (2 top and 2 bottom) and 1 at the side.

Step 4: Remove electrical connections. There is also a electrical connections on the left hand side of the manifold that are not shown in the pictures below.

Step 5: Remove the throttle cable

Step 6: Remove any hoses. There may be one or two more hoses that are not shown in the pictures.

Step7: Remove the 5 12mm bolts on the front of the manifold. There are also another 3 bolts at the back of the manifold

Step8: The manifold should now be free to remove, but be careful not to damage the gaskets if you are planning on reusing them. When you remove the manifold put some old rags into the intake holes because if drop something down them you might have to remove the head to take it out.

Step9: remove the timing belt cover, spark plug leads and rocker cover and replace tappets as you did for the front bank. Remember it is worth will changing the spark plus as well at this stage.

Step10: Refitting is the reverse of removal.